## A) Radar System Design and Signal Generation

Used Parameters:

| Parameter | Value |
| --- | --- |
| Carrier Frequency (Fc) | 77 GHz |
| PRI | 2 μs |
| Pulse Width (Tp) | 4.5 ns |
| Sampling Points (N) | 1000000 |
| SNR | -5 dB |

| Parameter | Formula | Result | Project Requirement |
| --- | --- | --- | --- |
| Max Unambiguous Range | Rmax=C*PRI2 | 299.8 m | Must be > 250m |
| Max Unambiguous Velocity | vmax=C*PRF4*Fc | 486.7 m/s | Realistic (-100 to 100) |
| Number of pulses | N=λ2*Vres*PRI | 2048 | Target velocity resolution < 0.5m/s |
| Pulse Width (Tp) | Tp=2* ∆RC | 4.5*10-9 | Target range resolution < 0.75m |

Reported Quantities:

Chosen Scenario:

| Parameter | Target 1 | Target 2 |
| --- | --- | --- |
| Range | 50m | 135m |
| Velocity | -20m/s | 35 m/s |

Transmitted and received signals plots:

Figure 1 Transmitted/Received signals plots

The left plot represents the transmitted pulse we zoomed in a lot to show that it in fact is a rectangular pulse as from far away it appears as if it’s a line. An important note for the second plot no attention was considered we picked a small value to simulate the effect as it caused a lot of issues when we tried to realistically model it when doing the next couple of tasks.

Range Time Plot:

Figure 2 Range Time Plot

## B) Radar System Design and Signal Generation

Received signal before any matched filter:

Figure 3 Noisy received signal plot

## C) Velocity Estimation using Doppler Analysis

Figure 4 Doppler Spectrum

| Target | Detected Range | True Range | Detected Velocities | True Velocities | Direction Classification |
| --- | --- | --- | --- | --- | --- |
| Target 1 | 50.01m | 50m | -19.96 m /s | 20.00 m /s | Approaching |
| Target 2 | 135.03m | 135m | 35 .17 m/s | 35 m /s | Receding |

D) Radar System Design and Signal Generation

Figure 5  Range Doppler Map